# West Doe - Regulatory Submittal Document Change Impact Report

Facilities covered: West Doe Compressor Station and Liquids Hub (3-25 and 2-25); West Doe Deepcut Expansion (4-25)

# 3-25 Regulatory Submittal Impact Table

| File No. | File Name | Revision | Date | Current Contents | Impact Status | Change Driver(s) | Required Action Summary | Evidence Basis |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1. By Tourmaline/03-25-080-15_SCHA Letter_Environmental Effects Assessment.pdf | Not stated | 2025-01-31 | Schedule A / SCHA environmental effects assessment correspondence. | Review Required | C3-SCA002, C3-SCA004 | Confirm whether environmental/effects statements, affected equipment, or consultation attachments still reflect retired 3-25 stabilization/SOC/dehydration scope. | PDF text includes SOC/NGL terms; SCA report changes project basis. |
| 2 | 1. By Tourmaline/100120203_C5+ and Inlet Compressor_PE Report_BRFN.pdf | Not stated | 2025-01-28 | PE notification/report package for C5+ and inlet compressor scope. | Review Required | C3-SCA001, C3-SCA002, C3-SCA003 | Review for affected C5+, inlet compressor, condensate treating, stabilization, and shared-utility basis before re-use. | File title; SCA evidence changes C5+/inlet compressor scope and downstream interfaces. |
| 3 | 1. By Tourmaline/100120203_C5+ and Inlet Compressor_PE Report_DRFN.pdf | Not stated | 2025-01-28 | PE notification/report package for C5+ and inlet compressor scope. | Review Required | C3-SCA001, C3-SCA002, C3-SCA003 | Same technical review as file 2; confirm recipient-specific transmittal text remains valid. | File title; SCA evidence changes C5+/inlet compressor scope and downstream interfaces. |
| 4 | 1. By Tourmaline/100120203_C5+ and Inlet Compressor_PE Report_HRFN.pdf | Not stated | 2025-01-28 | PE notification/report package for C5+ and inlet compressor scope. | Review Required | C3-SCA001, C3-SCA002, C3-SCA003 | Same technical review as file 2; confirm recipient-specific transmittal text remains valid. | File title; SCA evidence changes C5+/inlet compressor scope and downstream interfaces. |
| 5 | 1. By Tourmaline/100120203_C5+ and Inlet Compressor_PE Report_MLIB.pdf | Not stated | 2025-01-28 | PE notification/report package for C5+ and inlet compressor scope. | Review Required | C3-SCA001, C3-SCA002, C3-SCA003 | Same technical review as file 2; confirm recipient-specific transmittal text remains valid. | File title; SCA evidence changes C5+/inlet compressor scope and downstream interfaces. |
| 6 | 1. By Tourmaline/100120203_C5+ and Inlet Compressor_PE Report_SFN.pdf | Not stated | 2025-01-28 | PE notification/report package for C5+ and inlet compressor scope. | Review Required | C3-SCA001, C3-SCA002, C3-SCA003 | Same technical review as file 2; confirm recipient-specific transmittal text remains valid. | File title; SCA evidence changes C5+/inlet compressor scope and downstream interfaces. |
| 7 | 1. By Tourmaline/100120203_C5+ and Inlet Compressor_PE Report_WMFN.pdf | Not stated | 2025-01-28 | PE notification/report package for C5+ and inlet compressor scope. | Review Required | C3-SCA001, C3-SCA002, C3-SCA003 | Same technical review as file 2; confirm recipient-specific transmittal text remains valid. | File title; SCA evidence changes C5+/inlet compressor scope and downstream interfaces. |
| 8 | 1. By Tourmaline/2024 Tourmaline Fugitive Emission Management Plan (Signed).pdf | Signed | 2023-10-02 | Corporate fugitive emission management plan. | No Change Expected | None | No project-specific revision identified; keep as corporate supporting plan unless regulatory reviewer requests current signed version. | Targeted text has generic facility terms but no material SCA contradiction. |
| 9 | 1. By Tourmaline/3-25 West Doe Project Schedule.pdf | Not stated | 2025-02-03 | One-page project schedule/objective summary. | Change Required | C3-SCA001, C3-SCA002 | Update scope/objective text to remove 3-25 dehydration/stabilization/SOC/heat-medium assumptions and align timing with transferred 4-25 stabilization scope. | PDF text includes "dehydration"; SCA-001/SCA-002 retire that local scope. |
| 10 | 1. By Tourmaline/BCER Project Description Form - 3-25 West Doe (2025-01-31).pdf | Not stated | 2025-01-31 | BCER project description form for 3-25. | Change Required | C3-SCA001 through C3-SCA006 | Re-issue project description with current equipment list, process narrative, shared utilities, incinerator/flare boundary, LACT OUT, and 4-25 stabilization/SOC interface. | PDF text hits stabilizer/SOC/incinerator/instrument air; SCA report changes those bases. |
| 11 | 1. By Tourmaline/Tourmaline Plant Supervision Summary.pdf | Not stated | 2017-07-05 | General plant supervision summary. | No Change Expected | None | No SCA-driven content change identified. | Generic operations support document; no targeted project-scope contradiction found. |
| 12 | 1. By Tourmaline/Tourmaline Sand Management Plan - Revisions July 26, 2017.pdf | Revisions July 26, 2017 | 2021-10-12 | General sand management plan. | No Change Expected | None | No SCA-driven content change identified. | Generic plan; targeted text only shows incidental TEG term. |
| 13 | 1. By Tourmaline/West Doe Gathering (2025-02-03).pdf | Not stated | 2025-02-03 | West Doe gathering-system context document. | Review Required | C3-SCA002, D4-SCA001 | Review system schematic/narrative for stabilization relocation, NGL terminology, and 3-25 VRU discharge to 4-25 SOC suction. | PDF text hits stabilization/NGL/LPG; cross-facility SCA changes gathering/process interfaces. |
| 14 | BFD-242510-100_200_300 (3-25 Doe)_Block Flow Diagram_rA.pdf | rA | 2024-11-22 | Block flow diagram. | Change Required | C3-SCA001 through C3-SCA006 | Revise blocks and interfaces for retired 3-25 dehydration/stabilizer/SOC/heat-medium, non-regenerative treating, VRU to 4-25 SOC, shared IA/fuel gas/power, and incinerator/flare boundary. | Inferred from drawing scope; targeted SCA evidence directly changes represented process blocks. |
| 15 | CIV-242510-5001 & 5002 (3-25 Doe)_Plot Plan_rC.pdf | rC | 2024-11-22 | Civil plot plan set. | Change Required | C3-SCA001 through C3-SCA006 | Remove or relabel retired units/plot reservations, update incinerator/shared flare location/interface, LACT boundary, heat-medium/instrument-air assumptions, and affected package layout. | PDF text hits dehydration/stabilizer/SOC/heat medium/incinerator/LACT/IA; drawing impact inferred from scope. |
| 16 | MFS-242510 (3-25 Doe)_P&ID_rA.pdf | rA | 2024-12-10 | P&ID package. | Change Required | C3-SCA001 through C3-SCA006 | Revise process, utility, relief, control, shutdown, and interface sheets for all retired/relocated units and new shared utility/VRU/incinerator bases. | Inferred from drawing scope and targeted SCA process/interface changes. |
| 17 | MSS-242510-1001 (3-25 Doe)_Metering Schematic_rA.pdf | rA | 2024-12-06 | Metering schematic. | Review Required | C3-SCA001, C3-SCA003 | Confirm NRM connector and LACT OUT basis are reflected; revise only if metering/transfer points still imply on-site 3-25 LACT units. | SCA-001/SCA-003 change LACT and pipeline destination basis. |
| 18 | PFD-242510 (3-25 Doe)_Process Flow Diagram_rA.pdf | rA | 2024-11-22 | Process flow diagram package. | Change Required | C3-SCA001 through C3-SCA006 | Update process flow paths, material interfaces, utility ties, and removed equipment blocks; add/clarify 4-25 SOC and 4-25 IA interfaces. | Inferred from drawing scope; targeted SCA evidence changes represented process flows, with related text hits in linked reports. |
| 19 | W242510-CIV-REP-000001-001 (3-25 Doe)_Surface Water Management Plan_rA.pdf | rA | 2023-09-27 | Surface water management plan. | Review Required | C3-SCA001, C3-SCA002 | Check drainage/containment areas for retired dehydration/stabilizer/heat-medium/LACT units and added/retained incinerator/shared flare areas. | PDF text hits retired terms; physical layout changes may affect drainage assumptions. |
| 20 | W242510-ELC-REP-000001-001 (3-25 Doe)_Light Assessment_rA.pdf | rA | 2024-11-05 | Light assessment. | Review Required | C3-SCA001, C3-SCA002 | Update project description and lighting inventory if retired units or retained incinerator/flare/shared utility equipment change lighting sources. | PDF text contains old dehydration/stabilization/LACT/heat-medium narrative. |
| 21 | W242510-ELC-REP-000002-001 (3-25 Doe)_Ground Permissive at Truck-Out_rA.pdf | rA | 2024-11-12 | Ground permissive at truck-out philosophy. | Review Required | C3-SCA001, C3-SCA003 | Confirm truck-out scope and NRM LACT interface remain correct after LACT OUT and 480 V dry-type transformer decision. | PDF text hits LACT/truck-out/ground permissive; SCA changes LACT boundary. |
| 22 | W242510-PRC-DBM-000001-001 (3-25 Doe)_DBM_rA.pdf | rA | 2024-12-17 | Design basis memorandum. | Change Required | C3-SCA001 through C3-SCA006 | Rewrite DBM sections for removed/retired scope, process selection, shared utilities, 4-25 interfaces, flare/incinerator basis, horsepower, package roster, and supersession notes. | PDF text hits every major changed term; SCA report is DBM amendment basis. |
| 23 | W242510-PRC-HMB-000001-001 (3-25 Doe)_HMB_rA.pdf | rA | 2024-11-22 | Heat and material balance. | Change Required | C3-SCA001, C3-SCA002, C3-SCA006 | Recalculate HMB around removed stabilization/SOC/heat-medium/dehydration, non-regenerative treating, VRU discharge, and shared IA demands where represented. | PDF text hits SOC/incinerator/LACT/VRU/disulphide/spent caustic. |
| 24 | W242510-PRC-REG-000001-001 (3-25 Doe)_Emissions Summary_rA.pdf | rA | 2024-12-02 | Emissions summary. | Change Required | C3-SCA001 through C3-SCA006 | Recalculate emissions for removed heat-medium/stabilizer/SOC/dehydration sources, changed incinerator/feed basis, VRU routing, and shared utilities. | PDF text hits stabilizer/SOC/heat-medium/incinerator/flare/VRU. |
| 25 | W242510-PRC-REP-000001-001 (3-25 Doe)_Non-Flare Connected PSV Summary_rA.pdf | rA | 2023-09-13 | Non-flare connected PSV summary. | Change Required | C3-SCA001, C3-SCA002, C3-SCA003 | Remove retired equipment PSV entries, update relief/non-flare classification, and confirm current inlet separator/caustic/flare boundary PSV basis. | PDF text hits retired terms and flare/IA/LACT context. |
| 26 | W242510-PRC-REP-000002-001 (3-25 Doe)_Inlet Separator Summary_rA.pdf | rA | 2024-11-04 | Inlet separator summary. | Change Required | C3-SCA002, C3-SCA003 | Update to twin 9 ft x 40 ft horizontal 3-phase basis, plot for third as applicable, downstream 4-25 stabilization/SOC context, and current interface conditions. | SCA-002 changed separator sizing; PDF title identifies the affected inlet separator summary. |
| 27 | W242510-PRC-REP-000003-001 (3-25 Doe)_Prime Mover List_rA.pdf | rA | 2024-11-06 | Prime mover list. | Change Required | C3-SCA001, C3-SCA002, C3-SCA003, C3-SCA006 | Remove or reclassify retired prime movers, update inlet-compressor drive/VFD basis, standby generator strategy, and local IA/heat-medium equipment. | PDF text hits heat medium/LACT/VFD/VRU; SCA changes prime mover assumptions. |
| 28 | W242510-PRC-REP-000004-001 (3-25 Doe)_Facility Shutdown & Blowdown Philosophy_rA.pdf | rA | 2024-11-26 | Shutdown and blowdown philosophy. | Change Required | C3-SCA001 through C3-SCA006 | Update shutdown/blowdown scenarios, cause/effect assumptions, relief/blowdown destination, retired units, shared IA/power, and 4-25 SOC interface. | PDF text hits all major changed systems; flare/blowdown packages affected. |
| 29 | W242510-PRC-REP-000005-001 (3-25 Doe)_Flare Stack Data Submission_rA.pdf | rA | 2024-11-26 | Flare stack data submission. | Change Required | C3-SCA001, C3-SCA002, C3-SCA003 | Recalculate flare/incinerator loads and update shared flare/incinerator boundary, retired relief sources, and non-regenerative caustic/incinerator feed basis. | PDF text hits caustic regeneration, incinerator, flare, depropanizer/LPG, stabilizer/SOC. |
| 30 | W242510-PRC-REP-000007-001 (3-25 Doe)_DEOS_rB.pdf | rB | 2024-11-12 | DEOS report. | Review Required | C3-SCA001, C3-SCA002, C3-SCA003 | Confirm whether DEOS is limited to retained gas dehydration or incorrectly includes retired condensate dehydration/caustic-regeneration/incinerator assumptions. | PDF text hits dehydration/SOC/incinerator/flare/VRU. |
| 31 | W242510-PRC-REP-000008-001 (3-25 Doe)_Safety and Loss Management_rA.pdf | rA | 2023-09-27 | Safety and loss management report. | Review Required | C3-SCA001 through C3-SCA006 | Update safety narrative if retired/relocated units, shared utilities, flare/incinerator, or shutdown/blowdown changes alter safeguards or responsibilities. | PDF text includes stale process narrative; SCA changes hazardous systems. |
| 32 | W242510-PRJ-LST-000003-001 (3-25 Doe)_HAZMAT List_rA.pdf | rA | 2024-11-18 | Hazardous materials list. | Change Required | C3-SCA001, C3-SCA002, C3-SCA006 | Remove/adjust chemicals tied to retired condensate dehydration, heat-medium and local IA systems; confirm spent caustic/DSO basis under non-regenerative treating. | PDF text hits heat medium, TEG, spent caustic, IA dryer desiccant. |
| 33 | W242510-PRJ-QAP-000001-001 (3-25 Doe)_QAP & Validation Report_rA.pdf | rA | 2023-10-13 | QAP and validation report. | Review Required | C3-SCA004, C3-SCA005 | Confirm validation package lists and references include the SCA-004 package roster and SCA-005 retired-KTY cleanup where relevant. | SCA-004/SCA-005 add/clean derivative package structures. |
| 34 | W242510-PRJ-REP-000001-001 (3-25 Doe)_Secondary Containment Plan_rA.pdf | rA | 2023-09-27 | Secondary containment plan. | Change Required | C3-SCA001, C3-SCA002 | Update containment volumes/areas and material list for removed units, LACT boundary, non-regenerative caustic, spent caustic/DSO, and retained tanks. | PDF text hits dehydration/stabilizer/SOC/heat-medium/incinerator/LACT/spent caustic. |
| 35 | W242510-PRJ-REP-000002-001 (3-25 Doe)_Adopted Codes & Standards_rA.pdf | rA | 2024-11-04 | Adopted codes and standards. | Review Required | C3-SCA001 through C3-SCA006 | Review standards list for retired package standards and added/shared package standards; update if the document enumerates equipment-specific code bases. | PDF text includes stale scope terms; package roster changed. |
| 36 | W242510-PRJ-REP-000003-001 (3-25 Doe)_Facility Security and Fire Protection_rA.pdf | rA | 2024-11-04 | Facility security and fire protection. | Review Required | C3-SCA001 through C3-SCA006 | Confirm fire/security coverage reflects current equipment layout, deleted units, shared incinerator/flare responsibility, and tank/chemical changes. | PDF text includes stale process terms; physical hazard inventory changed. |
| 37 | W242510-PRJ-REP-000004-001(3-25 Doe)_HAZID Study Report_rA.pdf | rA | 2024-12-11 | HAZID study report. | Change Required | C3-SCA001 through C3-SCA006 | Reconcile HAZID nodes/scenarios and referenced documents with retired units, new interfaces, non-regenerative treating, shared flare/incinerator, and changed utilities. | PDF text references DEOS, inlet separator, flare data, LPG, spent caustic, ground permissive. |
| 38 | W242510-PRJ-REP-000005-001 (3-25 Doe)_Prelim Consequence Assessment_rA.pdf | rA | 2023-10-13 | Preliminary consequence assessment. | Change Required | C3-SCA001 through C3-SCA006 | Re-run scenario basis for removed/relocated equipment, changed relief/incinerator loads, changed hazardous inventory, and current shutdown/blowdown assumptions. | PDF text includes old dehydration/stabilization/LACT/heat-medium narrative. |
| 39 | W242510-PRJ-REP-000006-001 (3-25 Doe)_Noise Impact Assessment_r2.pdf | r2 | 2025-01-14 | Noise impact assessment. | Change Required | C3-SCA001 through C3-SCA006, D4-SCA001 | Rebuild noise model inventory to remove retired local units, update 4-25 shifted sources/interfaces, IA consolidation, incinerator location, and current equipment counts. | PDF text contains old 3-25 stabilizer/heat-medium/IA/LACT and 4-25 VRU/incinerator source entries. |

# 4-25 Regulatory Submittal Impact Table

| File No. | File Name | Revision | Date | Current Contents | Impact Status | Change Driver(s) | Required Action Summary | Evidence Basis |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 006810-NIA-000 (4-25 Doe)_Noise Impact Assessment_r0.pdf | r0 | 2023-11-30 | Noise impact assessment. | Change Required | D4-SCA001 through D4-SCA006, C3-SCA002 | Rebuild acoustic inventory and scenarios for LPG-to-NGL, depropanizer removal, stabilizer/SOC consolidation, IA consolidation, heat medium changes, and shared incinerator. | PDF text hits LPG/NGL/depropanizer/stabilizer/SOC/IA/heat medium/incinerator. |
| 2 | 1. By Tourmaline/02-25-080-15W6_Revision Letter_Pre-Con Env Assessment_26Sep2023.pdf | Revision letter | 2023-09-26 | Pre-construction environmental assessment revision letter. | Review Required | D4-SCA001, D4-SCA003 | Confirm environmental revision letter remains accurate after NGL/current-scope and depropanizer decisions. | PDF text hits SOC/NGL/TEG; SCA changes current facility scope. |
| 3 | 1. By Tourmaline/2-25 Project Description Form Final Draft_V2-20231102.pdf | V2 | 2023-11-02 | Project description form. | Change Required | D4-SCA001, D4-SCA002, D4-SCA006 | Update project description for NGL rather than LPG, depropanizer removal, current stabilizer/SOC scope, shared incinerator, and instrument air serving both facilities. | PDF text hits LPG/flare/truck-out; SCA changes scope. |
| 4 | 1. By Tourmaline/2024 Tourmaline Fugitive Emission Management Plan (Al Signed).pdf | Signed | 2023-10-02 | Corporate fugitive emission management plan. | No Change Expected | None | No project-specific revision identified; retain unless a newer signed corporate plan is required. | Generic corporate plan; no material SCA contradiction found. |
| 5 | 1. By Tourmaline/8-33 to West Doe Map.pdf | Not stated | 2023-09-28 | Route/context map. | No Change Expected | None | No SCA-driven map change identified. | No targeted scope-change contradiction found. |
| 6 | 1. By Tourmaline/Tourmaline Plant Supervision Plan.pdf | Not stated | 2017-07-05 | Plant supervision plan. | No Change Expected | None | No SCA-driven content change identified. | Generic operations plan. |
| 7 | 1. By Tourmaline/Tourmaline Sand Management Plan - Revisions July 26, 2017.pdf | Revisions July 26, 2017 | 2021-10-12 | Sand management plan. | No Change Expected | None | No SCA-driven content change identified. | Generic plan; targeted text only shows incidental TEG term. |
| 8 | 1. By Tourmaline/West Doe Gas Plant Proliferation Review.pdf | Not stated | 2017-06-23 | Gas plant proliferation review. | Review Required | D4-SCA001, C3-SCA002 | Confirm proliferation/expansion conclusions remain valid with 4-25 now receiving 3-25 stabilization/SOC and IA service. | Document role; SCA evidence changes cross-facility development basis. |
| 9 | 1. By Tourmaline/West Doe Gathering System Schematic.pdf | Not stated | 2023-10-05 | Gathering system schematic. | Review Required | D4-SCA001, C3-SCA002 | Review schematic for stabilization consolidation, NGL terminology, and 3-25 VRU-to-4-25 SOC interface. | PDF text hits stabilization/NGL; SCA changes gathering/process routing. |
| 10 | 1. By Tourmaline/West Doe Project Schedule.pdf | Not stated | 2023-10-17 | Project schedule. | Review Required | D4-SCA001 through D4-SCA006 | Confirm milestone/scope language remains current after depropanizer removal, package roster changes, and shared utility decisions. | Targeted SCA package and scope evidence may affect schedule content. |
| 11 | 100118719_FAC_2 25-80-15W6 (4-25 Doe)_PE Report.pdf | Not stated | 2023-10-31 | Facility PE report. | Review Required | D4-SCA001 through D4-SCA006 | Engineering sign-off package should be checked for old LPG/depropanizer, heat-medium, IA, incinerator, and stabilizer/SOC assumptions. | File role; targeted SCA evidence changes the engineered facility basis. |
| 12 | BFD-235633-100_200_5100 (4-25 Doe)_Block Flow Diagram_rC.pdf | rC | 2023-10-26 | Block flow diagram. | Change Required | D4-SCA001 through D4-SCA006 | Revise process blocks for NGL current scope, depropanizer removal, non-regenerative treating, IA consolidation, heat medium, mole sieve, stabilizer/SOC, and shared incinerator. | Inferred from drawing scope and targeted SCA process changes. |
| 13 | CIV-235633-5001 (4-25 Doe)_Plot Plan_rK.pdf | rK | 2024-10-24 | Civil plot plan sheet. | Change Required | D4-SCA001 through D4-SCA006 | Update layout for removed depropanizer/LPG future scope, current NGL packages, stabilizer/SOC consolidation, IA building, heat-medium configuration, and incinerator interface. | Inferred from drawing scope and targeted SCA package/equipment changes. |
| 14 | CIV-235633-5002 (4-25 Doe)_Plot Plan_rK.pdf | rK | 2025-04-21 | Civil plot plan package. | Change Required | D4-SCA001 through D4-SCA006 | Same plot-plan update as file 13; also check current rK sheet set for late partial updates against SCA-006 IA basis. | PDF text hits stabilizer/SOC/heat medium/incinerator/IA/LPG/depropanizer/mole sieve. |
| 15 | MFS-235633 (4-25 Doe)_P&ID_rA.pdf | rA | 2023-10-27 | P&ID package. | Change Required | D4-SCA001 through D4-SCA006 | Revise process, utility, relief, controls, and interface sheets for all current NGL, stabilizer/SOC, heat medium, IA, incinerator, and electrical decisions. | Inferred from drawing scope and targeted SCA process/interface changes. |
| 16 | MSS-235633-1001 (4-25 Doe)_Metering Schematic_rB.pdf | rB | 2025-04-21 | Metering schematic. | Review Required | D4-SCA001, D4-SCA003 | Confirm product/metering labels reflect NGL rather than LPG/C3-C4/depropanizer framing and no retired product stream remains. | SCA normalizes LPG to NGL and removes depropanizer. |
| 17 | PFD-235633 (4-25 Doe)_Process Flow Diagram_rB.pdf | rB | 2023-10-26 | Process flow diagram package. | Change Required | D4-SCA001 through D4-SCA006 | Update all flow paths and balances for NGL, removed depropanizer, stabilizer/SOC consolidation, inlet HEX, heat medium, TEG/mole sieve, IA, and incinerator basis. | Inferred from drawing scope and targeted SCA process changes. |
| 18 | W235633-CIV-REP-000001-001 (4-25 Doe)_Surface Water Management Plan_rB.pdf | rB | 2023-09-27 | Surface water management plan. | Review Required | D4-SCA001, D4-SCA004 | Check drainage/containment assumptions for current NGL/stabilizer/IA/heat-medium package layout and removed LPG/depropanizer scope. | PDF text hits LPG/NGL/heat medium/mole sieve; physical layout changed. |
| 19 | W235633-ELC-REP-000001-001 (4-25 Doe)_Light Assessment_rC.pdf | rC | 2023-09-26 | Light assessment. | Review Required | D4-SCA001, D4-SCA004, D4-SCA006 | Update lighting inventory and project description if package layout or current equipment differs from the submitted assessment. | PDF text hits LPG/NGL/heat medium/mole sieve and old process narrative. |
| 20 | W235633-ELC-REP-000002-001 (4-25 Doe)_Ground Permissive at Truck-Out_rB.pdf | rB | 2023-09-26 | Ground permissive at truck-out philosophy. | Review Required | D4-SCA001, D4-SCA003 | Confirm truck-out/product terminology and product streams reflect NGL current scope and not LPG/depropanizer framing. | PDF text hits LPG/NGL/truck-out; product scope changed. |
| 21 | W235633-PRC-DBM-000001-001 (4-25 Doe)_Design Basis Memorandum_rA.pdf | rA | 2023-10-23 | Design basis memorandum. | Change Required | D4-SCA001 through D4-SCA006 | Rewrite DBM sections for current NGL, depropanizer removal, non-regenerative treating, stabilizer/SOC scope, heat medium, TEG/mole sieve, IA, incinerator, electrical, and package roster. | PDF text hits every major changed term; SCA report is DBM amendment basis. |
| 22 | W235633-PRC-HMB-000001-001 (4-25 Doe)_HMB Winter_rC.pdf | rC | 2023-10-24 | Winter HMB. | Change Required | D4-SCA001 through D4-SCA006 | Recalculate winter HMB for NGL, no depropanizer, non-regenerative treating, changed heat medium, TEG/mole sieve, IA demand, and incinerator feed. | PDF text hits LPG, SOC, incinerator, VRU, TEG, spent caustic. |
| 23 | W235633-PRC-HMB-000002-001 (4-25 Doe)_HMB Summer_rC.pdf | rC | 2023-10-24 | Summer HMB. | Change Required | D4-SCA001 through D4-SCA006 | Same as winter HMB, with summer operating basis and heat/refrigeration balance reviewed separately. | PDF text hits LPG, SOC, incinerator, VRU, TEG, spent caustic. |
| 24 | W235633-PRC-REG-000001-001 (4-25 Doe)_Emissions Summary_rB.pdf | rB | 2024-10-23 | Emissions summary. | Change Required | D4-SCA001 through D4-SCA006 | Recalculate emissions for removed depropanizer/LPG equipment, NGL treating, heat medium, IA, incinerator, stabilizer/SOC, flare, and equipment roster. | PDF text hits stabilizer/SOC/heat medium/incinerator/LPG/depropanizer/mole sieve. |
| 25 | W235633-PRC-REP-000001-001 (4-25 Doe)_Non-Flare Connected PSV Summary_rA.pdf | rA | 2023-09-29 | Non-flare connected PSV summary. | Change Required | D4-SCA001 through D4-SCA006 | Remove retired depropanizer/LPG PSV basis and update current NGL, stabilizer/SOC, heat medium, IA, and relief/non-flare classification. | PDF text hits depropanizer/LPG/NGL/heat medium/IA/PSV-relevant terms. |
| 26 | W235633-PRC-REP-000002-001 (4-25 Doe)_Inlet Separator Summary.pdf | Not stated | 2023-09-29 | Inlet separator summary. | Change Required | D4-SCA001, D4-SCA005 | Update to current two 9 ft x 40 ft installed separators, future third plot, and inlet-separator HEX / MPFF preheat relationship. | SCA-001 changed separator sizing; SCA-005 registers the inlet-separator HEX subject. |
| 27 | W235633-PRC-REP-000003-001 (4-25 Doe)_Prime Mover List_rC.pdf | rC | 2023-03-01 | Prime mover list. | Change Required | D4-SCA001, D4-SCA006 | Update drivers/motors for current stabilizer/SOC, heat medium, TEG/mole sieve, instrument air, VFD, capacitor-bank removal, and standby power basis; remove depropanizer/LPG equipment if present. | PDF text hits LPG/NGL/stabilizer/heat medium/VFD/mole sieve. |
| 28 | W235633-PRC-REP-000004-001 (4-25 Doe)_Plant Shutdown & Blowdown Philosophy_rB.pdf | rB | 2023-09-29 | Shutdown and blowdown philosophy. | Change Required | D4-SCA001 through D4-SCA006 | Update shutdown/blowdown causes, relief destinations, current NGL process, depropanizer removal, heat medium, IA, and incinerator/shared interface responsibilities. | PDF text hits depropanizer/LPG/NGL/heat medium/incinerator/IA. |
| 29 | W235633-PRC-REP-000005-001 (4-25 Doe)_Flare Stack Data Submission_rA.pdf | rA | 2023-10-27 | Flare stack data submission. | Change Required | D4-SCA001, D4-SCA002, D4-SCA003 | Recalculate flare/incinerator loads and feed basis; remove caustic-regeneration/depropanizer relief bases; identify 3-25 physical incinerator location. | PDF text hits caustic regeneration/incinerator/flare/LPG/depropanizer/mole sieve. |
| 30 | W235633-PRC-REP-000007-001 (4-25 Doe)_DEOS_rB.pdf | rB | 2023-10-25 | DEOS report. | Review Required | D4-SCA001, D4-SCA005 | Confirm whether DEOS is limited to retained dehydration equipment and does not carry obsolete LPG/depropanizer/non-current incinerator assumptions. | PDF text hits dehy/SOC/incinerator/flare/VRU/TEG. |
| 31 | W235633-PRC-REP-000008-001 (4-25 Doe)_Safety & Loss Management_rA.pdf | rA | 2023-09-27 | Safety and loss management report. | Review Required | D4-SCA001 through D4-SCA006 | Update safety narrative if NGL/depropanizer removal, shared incinerator, IA, heat medium, or shutdown/blowdown changes alter safeguards or responsibilities. | PDF text hits LPG/NGL/stabilization/SOC/heat medium/flare/mole sieve. |
| 32 | W235633-PRJ-LST-000003-001 (4-25 Doe)_HAZMAT List_rC.pdf | rC | 2023-09-27 | Hazardous materials list. | Change Required | D4-SCA001 through D4-SCA006 | Replace LPG/depropanizer-related chemicals with current NGL/DSO/spent caustic basis; update heat medium, IA dryer, TEG/mole sieve consumables. | PDF text hits LPG/NGL/heat medium/IA/TEG/mole sieve/spent caustic. |
| 33 | W235633-PRJ-QAP-000001-001 (4-25 Doe)_QAP & Validation Report_rA.pdf | rA | 2023-10-19 | QAP and validation report. | Review Required | D4-SCA004, D4-SCA005 | Confirm validation package lists and references include SCA-004 package roster and SCA-005 content remediation. | SCA-004/SCA-005 add/clean derivative package structures. |
| 34 | W235633-PRJ-REP-000001-001 (4-25 Doe)_Secondary Containment Plan_rB.pdf | rB | 2023-09-27 | Secondary containment plan. | Change Required | D4-SCA001 through D4-SCA006 | Update containment basis for NGL instead of LPG, no depropanizer, non-regenerative caustic, DSO/spent caustic, heat medium, and package layout changes. | PDF text hits depropanizer/LPG/NGL/heat medium/incinerator/spent caustic. |
| 35 | W235633-PRJ-REP-000002-001 (4-25 Doe)_Adopted Codes & Standards_rB.pdf | rB | 2023-09-27 | Adopted codes and standards. | Review Required | D4-SCA001 through D4-SCA006 | Review standards list for retired depropanizer/LPG standards and current NGL, heat-medium, mole sieve, IA, incinerator, and electrical package standards. | PDF text includes stale scope terms and affected package categories. |
| 36 | W235633-PRJ-REP-000003-001 (4-25 Doe)_Facility Security & Fire Protection_rC.pdf | rC | 2023-09-27 | Facility security and fire protection. | Review Required | D4-SCA001 through D4-SCA006 | Confirm fire/security coverage reflects current equipment layout, hazardous inventory, and shared incinerator/utility responsibility. | PDF text includes LPG/NGL/heat medium/flare/mole sieve terms. |
| 37 | W235633-PRJ-REP-000004-001 (4-25 Doe)_HAZID Study Report_r0.pdf | r0 | 2023-10-05 | HAZID study report. | Change Required | D4-SCA001 through D4-SCA006 | Reconcile HAZID nodes/scenarios and referenced documents with depropanizer removal, NGL basis, non-regenerative caustic, IA, heat medium, mole sieve, and shared incinerator. | PDF text hits every major changed term including depropanizer/LPG/incinerator/IA/heat medium/mole sieve. |
| 38 | W235633-PRJ-REP-000005-001 (4-25 Doe)_Prelim Consequence Assessment_rA.pdf | rA | 2023-10-13 | Preliminary consequence assessment. | Change Required | D4-SCA001 through D4-SCA006 | Re-run consequence basis for removed/changed equipment, changed inventory, shared incinerator, relief/blowdown, and utility interfaces. | PDF text includes LPG/NGL/heat medium/mole sieve/stabilization terms. |
| 39 | W235633-PRJ-REP-000006-001 (4-25 Doe)_Design Change Summary_rB.pdf | rB | 2023-09-27 | Design change summary. | Change Required | D4-SCA001 through D4-SCA006 | Rewrite change summary to capture all SCA decisions: depropanizer out, NGL current scope, stabilizer/SOC consolidation, instrument air serving both facilities, heat medium, HEX, mole sieve, incinerator, and standby/VFD/capacitor electrical changes. | PDF text hits depropanizer/LPG/NGL/heat medium/incinerator/LACT/plot-related terms. |

# 3-25 Detailed Document Change Notes

## 1. 03-25-080-15_SCHA Letter_Environmental Effects Assessment.pdf

Why this document is impacted: The assessment correspondence may rely on pre-SCA project descriptions or equipment lists.

Specific expected revisions: Review environmental effects, footprint, emissions/noise attachments, and consultation attachments for retired 3-25 stabilization/SOC/dehydration and current shared-incinerator basis.

Evidence used: PDF text extraction found SOC/NGL terminology; SCA-002 and SCA-004 changed process scope and package structure.

Review caveats: Regulatory counsel/environmental lead should decide whether a revision letter is enough.

## 2. 100120203_C5+ and Inlet Compressor_PE Report_BRFN.pdf

Why this document is impacted: This BRFN PE report is explicitly tied to C5+ and inlet compressor scope, which changed materially.

Specific expected revisions: Align the common technical content for non-regenerative treating, no 3-25 stabilizers/SOC/heat medium, inlet separator/compressor basis, VRU-to-4-25 SOC routing, and shared utilities.

Evidence used: The regulatory PDF title identifies C5+ and inlet compressor scope; C3-SCA001, C3-SCA002, and C3-SCA003 modify the affected scope basis.

Review caveats: Confirm whether recipient-specific consultation records require a revision letter or a full replaced attachment.

## 3. 100120203_C5+ and Inlet Compressor_PE Report_DRFN.pdf

Why this document is impacted: This DRFN PE report is explicitly tied to C5+ and inlet compressor scope, which changed materially.

Specific expected revisions: Align the common technical content for non-regenerative treating, no 3-25 stabilizers/SOC/heat medium, inlet separator/compressor basis, VRU-to-4-25 SOC routing, and shared utilities.

Evidence used: The regulatory PDF title identifies C5+ and inlet compressor scope; C3-SCA001, C3-SCA002, and C3-SCA003 modify the affected scope basis.

Review caveats: Confirm whether recipient-specific consultation records require a revision letter or a full replaced attachment.

## 4. 100120203_C5+ and Inlet Compressor_PE Report_HRFN.pdf

Why this document is impacted: This HRFN PE report is explicitly tied to C5+ and inlet compressor scope, which changed materially.

Specific expected revisions: Align the common technical content for non-regenerative treating, no 3-25 stabilizers/SOC/heat medium, inlet separator/compressor basis, VRU-to-4-25 SOC routing, and shared utilities.

Evidence used: The regulatory PDF title identifies C5+ and inlet compressor scope; C3-SCA001, C3-SCA002, and C3-SCA003 modify the affected scope basis.

Review caveats: Confirm whether recipient-specific consultation records require a revision letter or a full replaced attachment.

## 5. 100120203_C5+ and Inlet Compressor_PE Report_MLIB.pdf

Why this document is impacted: This MLIB PE report is explicitly tied to C5+ and inlet compressor scope, which changed materially.

Specific expected revisions: Align the common technical content for non-regenerative treating, no 3-25 stabilizers/SOC/heat medium, inlet separator/compressor basis, VRU-to-4-25 SOC routing, and shared utilities.

Evidence used: The regulatory PDF title identifies C5+ and inlet compressor scope; C3-SCA001, C3-SCA002, and C3-SCA003 modify the affected scope basis.

Review caveats: Confirm whether recipient-specific consultation records require a revision letter or a full replaced attachment.

## 6. 100120203_C5+ and Inlet Compressor_PE Report_SFN.pdf

Why this document is impacted: This SFN PE report is explicitly tied to C5+ and inlet compressor scope, which changed materially.

Specific expected revisions: Align the common technical content for non-regenerative treating, no 3-25 stabilizers/SOC/heat medium, inlet separator/compressor basis, VRU-to-4-25 SOC routing, and shared utilities.

Evidence used: The regulatory PDF title identifies C5+ and inlet compressor scope; C3-SCA001, C3-SCA002, and C3-SCA003 modify the affected scope basis.

Review caveats: Confirm whether recipient-specific consultation records require a revision letter or a full replaced attachment.

## 7. 100120203_C5+ and Inlet Compressor_PE Report_WMFN.pdf

Why this document is impacted: This WMFN PE report is explicitly tied to C5+ and inlet compressor scope, which changed materially.

Specific expected revisions: Align the common technical content for non-regenerative treating, no 3-25 stabilizers/SOC/heat medium, inlet separator/compressor basis, VRU-to-4-25 SOC routing, and shared utilities.

Evidence used: The regulatory PDF title identifies C5+ and inlet compressor scope; C3-SCA001, C3-SCA002, and C3-SCA003 modify the affected scope basis.

Review caveats: Confirm whether recipient-specific consultation records require a revision letter or a full replaced attachment.

## 9. 3-25 West Doe Project Schedule.pdf

Why this document is impacted: The schedule/objective text still references dehydration.

Specific expected revisions: Remove 3-25 condensate dehydration and local stabilization/SOC implications; identify any 4-25 dependency for stabilization/SOC interfaces if the schedule lists them.

Evidence used: PDF text extraction found "dehydration"; C3-SCA001 and C3-SCA002 retire local dehydration/stabilizer/SOC scope.

Review caveats: Confirm whether this is a regulatory schedule or informational attachment before re-issuing.

## 10. BCER Project Description Form - 3-25 West Doe (2025-01-31).pdf

Why this document is impacted: It is the primary regulatory project scope form and contains multiple superseded scope elements.

Specific expected revisions: Update equipment list, process narrative, phase description, shared utilities, incinerator/flare location and service, LACT OUT, and 4-25 stabilization/SOC interface.

Evidence used: PDF text hits stabilizer, SOC, incinerator, instrument air, inlet separator, truck-out; SCA-001 through SCA-006 change these bases.

Review caveats: BCER form fields may require a formal amendment rather than a narrative appendix.

## 13. West Doe Gathering (2025-02-03).pdf

Why this document is impacted: It contains gathering and process-interface context using stabilization/NGL/LPG terminology.

Specific expected revisions: Review 3-25/4-25 flow routes, product terminology, and stabilization/SOC routing; update if the schematic implies local 3-25 stabilization or obsolete LPG framing.

Evidence used: PDF text hits stabilization, NGL, and LPG; SCA-002 and D4-SCA001 modify the cross-facility routing.

Review caveats: Some gathering context may remain unchanged if it is upstream-only.

## 14. BFD-242510-100_200_300 (3-25 Doe)_Block Flow Diagram_rA.pdf

Why this document is impacted: Block diagrams are directly invalidated by removed and rerouted process blocks.

Specific expected revisions: Remove 3-25 condensate dehydration, stabilizer, SOC, heat-medium blocks; show non-regenerative treating, 4-25 SOC suction interface, 4-25 IA supply, shared flare/incinerator basis, and LACT OUT boundary.

Evidence used: SCA process decisions; drawing role and linked report text.

Review caveats: Drawing text extraction is limited; confirm visually against original CAD/PDF.

## 15. CIV-242510-5001 & 5002 (3-25 Doe)_Plot Plan_rC.pdf

Why this document is impacted: The plot plan still includes or references retired/changed physical equipment.

Specific expected revisions: Remove/flag retired local units and cancelled future condensate dehydration plot space; show retained incinerator/shared flare and current utility/package locations.

Evidence used: PDF text hits dehydration, stabilizer, SOC, heat medium, incinerator, LACT, instrument air, LPG/depropanizer terms; C3-SCA001 through C3-SCA006 alter the represented scope.

Review caveats: CAD layer/source drawing review is required for final layout change.

## 16. MFS-242510 (3-25 Doe)_P&ID_rA.pdf

Why this document is impacted: P&IDs carry the detailed process, utility, control, shutdown, and relief interfaces changed by the SCAs.

Specific expected revisions: Remove retired process systems, update caustic treating, VRU discharge, shared IA/fuel gas/power, flare/incinerator, relief, shutdown, and LACT boundary sheets.

Evidence used: The regulatory PDF is the 3-25 P&ID package; C3-SCA001 through C3-SCA006 alter systems shown on P&IDs.

Review caveats: Requires discipline drawing list and line-by-line P&ID revision, not only narrative edits.

## 17. MSS-242510-1001 (3-25 Doe)_Metering Schematic_rA.pdf

Why this document is impacted: Metering may still imply on-site LACT units or superseded transfer points.

Specific expected revisions: Confirm NRM NEBC Connector and LACT OUT routing; update metering nodes if LACT or product transfer boundaries changed.

Evidence used: The regulatory PDF is the 3-25 metering schematic; C3-SCA001 confirms LACT OUT and C3-SCA003 names the NRM NEBC Connector.

Review caveats: May become no-change if schematic is upstream/gas-only.

## 18. PFD-242510 (3-25 Doe)_Process Flow Diagram_rA.pdf

Why this document is impacted: The flow diagram must reflect retired and rerouted process systems.

Specific expected revisions: Update equipment blocks, stream routes, 4-25 SOC suction tie-in, shared utility arrows, inlet separator basis, and incinerator/flare streams.

Evidence used: The regulatory PDF is the 3-25 process flow diagram package; C3-SCA001 through C3-SCA006 alter represented process decisions.

Review caveats: Flow values must be reconciled with revised HMB before final drawing issue.

## 19. Surface Water Management Plan_rA.pdf

Why this document is impacted: Physical equipment and containment/drainage areas changed.

Specific expected revisions: Check runoff, grading, drainage and containment assumptions for deleted units, retained incinerator/flare, and current tank/package layout.

Evidence used: PDF text hits dehydration/stabilization/heat medium/LACT; SCA-001/SCA-002 alter physical scope.

Review caveats: Requires civil layout confirmation before classifying exact changes.

## 20. Light Assessment_rA.pdf

Why this document is impacted: The assessment narrative includes stale process/equipment scope and the lighting inventory may include deleted units.

Specific expected revisions: Remove or update references to local dehydration/stabilization/LACT/heat-medium systems; verify lighting sources for retained incinerator/shared utilities.

Evidence used: PDF text hits old scope terms; C3-SCA001 and C3-SCA002 remove or relocate several equipment bases that can affect the lighting inventory.

Review caveats: If lighting fixtures did not move, only narrative revisions may be required.

## 21. Ground Permissive at Truck-Out_rA.pdf

Why this document is impacted: The document is tied to truck-out and LACT-related electrical boundaries.

Specific expected revisions: Confirm truck-out scope, LACT OUT boundary, NRM interface, and 480 V dry-type transformer implications.

Evidence used: PDF text hits LACT/truck-out; SCA-001/SCA-003 change LACT and connector basis.

Review caveats: Final revision depends on whether the ground permissive applies only to retained truck-out systems.

## 22. W242510-PRC-DBM-000001-001 (3-25 Doe)_DBM_rA.pdf

Why this document is impacted: It is the central design basis and contains all major superseded scope families.

Specific expected revisions: Rewrite process, utility, flare/incinerator, electrical, package roster, equipment list, objective, and supersession sections.

Evidence used: PDF text hits every major SCA term; SCA report is DBM amendment basis.

Review caveats: DBM should be revised before dependent regulatory reports are finalized.

## 23. W242510-PRC-HMB-000001-001 (3-25 Doe)_HMB_rA.pdf

Why this document is impacted: Material balances change when stabilization/SOC/heat-medium/dehydration are removed or relocated.

Specific expected revisions: Remove retired units and recalculate streams around inlet separation, condensate treating, VRU, shared incinerator, and 4-25 SOC interface.

Evidence used: PDF text hits SOC/incinerator/LACT/VRU/spent caustic; SCA-002 reroutes flows.

Review caveats: Requires process engineering recalculation.

## 24. W242510-PRC-REG-000001-001 (3-25 Doe)_Emissions Summary_rA.pdf

Why this document is impacted: Emission sources and rates are directly affected by removed/changed equipment.

Specific expected revisions: Recalculate emissions inventory for retired heat-medium/stabilization/SOC/dehydration, updated incinerator feed, VRU routing, and shared utilities.

Evidence used: PDF text hits emissions-relevant changed equipment; C3-SCA001 through C3-SCA006 alter equipment, utility, flare/incinerator, and emissions bases.

Review caveats: Should follow updated PFD/HMB and flare/incinerator basis.

## 25. Non-Flare Connected PSV Summary_rA.pdf

Why this document is impacted: Relief devices tied to retired systems or changed flare boundaries may be obsolete.

Specific expected revisions: Delete retired equipment PSVs, update non-flare/flare-connected status, and verify inlet separator and caustic treating relief basis.

Evidence used: PDF text hits retired systems and flare/IA/LACT terms; C3-SCA001, C3-SCA002, and C3-SCA003 alter the PSV and relief-source basis.

Review caveats: Requires P&ID and relief-device list review.

## 26. Inlet Separator Summary_rA.pdf

Why this document is impacted: SCA-002 changes inlet separator sizing and downstream context.

Specific expected revisions: Update to twin 9 ft x 40 ft horizontal 3-phase separators, plot for a third if applicable, and current downstream stabilization/SOC interface.

Evidence used: The regulatory PDF is the 3-25 inlet separator summary; C3-SCA002 defines the changed separator basis and the PDF title identifies the affected separator document.

Review caveats: Confirm final separator design pressure/temperature against revised PFD/P&ID.

## 27. Prime Mover List_rA.pdf

Why this document is impacted: Prime movers and electrical starting strategies changed.

Specific expected revisions: Update inlet compressor horsepower/VFD basis, delete retired heat-medium/local IA items, and align standby generator strategy.

Evidence used: PDF text hits VFD/heat-medium/LACT/VRU; SCA-001/SCA-003 change VFD and horsepower.

Review caveats: Electrical studies may be needed for final motor/driver ratings.

## 28. Facility Shutdown & Blowdown Philosophy_rA.pdf

Why this document is impacted: Shutdown and blowdown logic depends on the current equipment and relief network.

Specific expected revisions: Remove retired units, update blowdown destinations, shared IA/power failure modes, incinerator/flare boundary, and VRU/SOC interface handling.

Evidence used: PDF text hits all major changed systems; C3-SCA001 through C3-SCA006 alter shutdown, blowdown, relief, utility, and interface assumptions.

Review caveats: Should be revised with P&ID and cause/effect updates.

## 29. Flare Stack Data Submission_rA.pdf

Why this document is impacted: Flare and incinerator loads are directly affected by retired units and non-regenerative caustic.

Specific expected revisions: Recalculate flare loads, remove retired relief sources, update shared incinerator location/feed/flow basis, and reflect non-regenerative caustic streams.

Evidence used: PDF text hits caustic regeneration/incinerator/flare/depropanizer/LPG terms; SCA-003 resolves flare boundary.

Review caveats: Requires relief/blowdown calculation package.

## 30. DEOS_rB.pdf

Why this document is impacted: It may mix retained gas dehydration with retired condensate dehydration or incinerator assumptions.

Specific expected revisions: Confirm scope of the DEOS; remove retired condensate-dehydration or caustic-regeneration references if present.

Evidence used: PDF text hits dehydration/SOC/incinerator/flare/VRU; C3-SCA001, C3-SCA002, and C3-SCA003 alter retained dehydration, incinerator, flare, and VRU context.

Review caveats: May be no-change if it only covers retained gas dehydration.

## 31. Safety and Loss Management_rA.pdf

Why this document is impacted: Hazard controls and responsibilities may change with deleted and shared systems.

Specific expected revisions: Review and update safeguards, shared-asset responsibility, emergency isolation, and process narrative.

Evidence used: PDF text has stale process narrative; SCA changes hazardous systems.

Review caveats: May depend on revised HAZID and shutdown/blowdown philosophy.

## 32. HAZMAT List_rA.pdf

Why this document is impacted: Chemical and waste inventories change with retired systems and non-regenerative treating.

Specific expected revisions: Remove heat-medium/dehydration/local IA consumables if no longer applicable; confirm spent caustic, DSO, TEG, and tank inventories.

Evidence used: PDF text hits spent caustic, heat medium, TEG, IA dryer desiccant; C3-SCA001, C3-SCA002, and C3-SCA006 alter the material inventory basis.

Review caveats: Final quantities require updated HMB and tank list.

## 33. QAP & Validation Report_rA.pdf

Why this document is impacted: The package/decomposition structure changed through SCA-004/SCA-005.

Specific expected revisions: Check validation references, package roster, and retired-KTY cleanup status.

Evidence used: The regulatory PDF is the 3-25 QAP and validation report; C3-SCA004 and C3-SCA005 change package roster and cleanup validation context.

Review caveats: May be an administrative revision only.

## 34. Secondary Containment Plan_rA.pdf

Why this document is impacted: Containment areas and inventories are affected by retired equipment and changed tank/chemical basis.

Specific expected revisions: Update containment areas, volumes, tank services, spent caustic/DSO handling, and deleted LACT/dehydration/heat-medium equipment.

Evidence used: PDF text hits dehydration/stabilizer/SOC/heat-medium/LACT/spent caustic; C3-SCA001 and C3-SCA002 alter containment-relevant equipment and material bases.

Review caveats: Requires final tank list and plot plan.

## 35. Adopted Codes & Standards_rA.pdf

Why this document is impacted: Equipment-specific codes may be stale after package roster changes.

Specific expected revisions: Review for retired-unit standards and added/current package standards.

Evidence used: SCA package roster changes and PDF stale terms.

Review caveats: May be no-change if document is generic and not package-specific.

## 36. Facility Security and Fire Protection_rA.pdf

Why this document is impacted: Physical layout, inventory, and shared incinerator/flare responsibility changed.

Specific expected revisions: Review firewater/extinguishing, separation, access, hazardous inventory, shared asset responsibility, and response assumptions.

Evidence used: PDF text includes stale scope terms; SCA changes physical hazard basis.

Review caveats: Requires updated plot and HAZMAT list.

## 37. HAZID Study Report_rA.pdf

Why this document is impacted: HAZID scenarios are based on superseded equipment and process interfaces.

Specific expected revisions: Update nodes, scenarios, recommendations, referenced documents, and risk register for all SCA decisions.

Evidence used: PDF references inlet separator, flare data, DEOS, LPG/spent caustic/ground permissive; SCA changes these bases.

Review caveats: Requires facilitated review or formal addendum.

## 38. Prelim Consequence Assessment_rA.pdf

Why this document is impacted: Consequence scenarios depend on current inventory, equipment, and relief/blowdown basis.

Specific expected revisions: Reassess scenarios for retired equipment, shared flare/incinerator, changed chemicals, and utility failure modes.

Evidence used: PDF text includes old dehydration/stabilization/LACT/heat-medium narrative; C3-SCA001 through C3-SCA006 alter consequence-relevant process, utility, and relief bases.

Review caveats: Should follow revised HAZID and flare/blowdown data.

## 39. Noise Impact Assessment_r2.pdf

Why this document is impacted: The noise model includes retired and shifted equipment sources.

Specific expected revisions: Remove retired 3-25 stabilizer/heat-medium/local IA/LACT sources, update VRU/compressor/incinerator/shared equipment, and reconcile 4-25 shifted sources.

Evidence used: PDF text lists 3-25 stabilizers, heat-medium, IA, LACT, VRU, 4-25 incinerator and VRU sources; C3-SCA001, C3-SCA002, and C3-SCA006 alter noise-source and utility assumptions.

Review caveats: Requires acoustic model update and regulatory noise specialist review.

# 4-25 Detailed Document Change Notes

## 1. 006810-NIA-000 (4-25 Doe)_Noise Impact Assessment_r0.pdf

Why this document is impacted: It contains many superseded equipment/source terms.

Specific expected revisions: Rebuild noise source inventory for NGL current scope, removed depropanizer, current stabilizer/SOC, heat medium, IA, mole sieve/TEG, and shared incinerator.

Evidence used: PDF text hits LPG/NGL/depropanizer/stabilizer/SOC/IA/heat medium/incinerator; D4-SCA001 through D4-SCA006 alter noise-source and process/equipment bases.

Review caveats: Requires acoustic model re-run.

## 2. 02-25-080-15W6_Revision Letter_Pre-Con Env Assessment_26Sep2023.pdf

Why this document is impacted: Environmental assessment revision may rely on pre-SCA 4-25 scope.

Specific expected revisions: Review for LPG/depropanizer/future-scope framing, NGL current scope, and shifted stabilization/SOC/IA assumptions.

Evidence used: PDF text hits SOC/NGL/TEG.

Review caveats: Environmental lead should decide whether a letter addendum is enough.

## 3. 2-25 Project Description Form Final Draft_V2-20231102.pdf

Why this document is impacted: Project description forms must match current process scope.

Specific expected revisions: Replace LPG/depropanizer/future framing with current NGL scope; add/confirm shared incinerator and instrument air serving both facilities.

Evidence used: PDF text hits LPG/flare/truck-out; SCA-001/SCA-002/SCA-006 change these bases.

Review caveats: Confirm BCER form version requirements.

## 8. West Doe Gas Plant Proliferation Review.pdf

Why this document is impacted: Facility proliferation conclusions may be affected by shifted/consolidated scope.

Specific expected revisions: Review conclusions against 4-25 receiving 3-25 stabilizer/SOC functions and consolidated IA.

Evidence used: The regulatory PDF is the West Doe gas plant proliferation review; D4-SCA001 and C3-SCA002 create cross-facility scope changes relevant to its role.

Review caveats: May remain unchanged if only high-level historic context.

## 9. West Doe Gathering System Schematic.pdf

Why this document is impacted: Gathering/process routing changed at the 3-25/4-25 interface.

Specific expected revisions: Update stabilization, NGL terminology, and 3-25 VRU to 4-25 SOC routing if shown.

Evidence used: PDF text hits stabilization/NGL; D4-SCA001 and C3-SCA002 alter the 4-25 NGL basis and the cross-facility stabilization/SOC interface.

Review caveats: Upstream gathering-only portions may not change.

## 10. West Doe Project Schedule.pdf

Why this document is impacted: Schedule may still reflect removed/renamed scope and package sequence.

Specific expected revisions: Review scope labels, package milestones, and dependencies for depropanizer removal, NGL current scope, and IA/stabilizer consolidation.

Evidence used: The regulatory PDF is the 4-25 project schedule; D4-SCA001 through D4-SCA006 alter package and sequencing assumptions.

Review caveats: Administrative schedule revision may be sufficient.

## 11. 100118719_FAC_2 25-80-15W6 (4-25 Doe)_PE Report.pdf

Why this document is impacted: It supports the engineered facility application and may contain superseded design basis.

Specific expected revisions: Review against all major 4-25 SCA changes before re-use.

Evidence used: The regulatory PDF is the 4-25 facility PE report; D4-SCA001 through D4-SCA006 alter the technical scope basis it supports.

Review caveats: Engineer-of-record should determine re-seal/re-issue requirements.

## 12. BFD-235633-100_200_5100 (4-25 Doe)_Block Flow Diagram_rC.pdf

Why this document is impacted: Block flow changed materially.

Specific expected revisions: Remove depropanizer/LPG blocks, show current NGL treating/storage/loading, stabilizer/SOC, heat medium, IA, mole sieve/TEG, and incinerator interfaces.

Evidence used: The regulatory PDF is the 4-25 block flow diagram; D4-SCA001 through D4-SCA006 alter represented process blocks and interfaces.

Review caveats: Confirm current balances before issuing.

## 13. CIV-235633-5001 (4-25 Doe)_Plot Plan_rK.pdf

Why this document is impacted: Current equipment layout changed or requires confirmation after multiple SCAs, and this plot-plan sheet is part of the physical layout control set.

Specific expected revisions: Remove or flag depropanizer/LPG future equipment if still shown, show current NGL/stabilizer/SOC/IA/heat-medium package locations, and clarify the shared incinerator relationship to the 3-25 location.

Evidence used: The regulatory PDF is a plot-plan sheet; D4-SCA001 through D4-SCA006 alter equipment roster, utility, and incinerator layout assumptions. Text extraction was sparse, so drawing-role inference is used.

Review caveats: CAD source review is required before deciding whether sheet 5001 needs graphical revision or only supersession notation.

## 14. CIV-235633-5002 (4-25 Doe)_Plot Plan_rK.pdf

Why this document is impacted: Current equipment layout changed or requires confirmation after multiple SCAs, and this plot-plan sheet includes extracted stale process and utility terms.

Specific expected revisions: Remove or flag depropanizer/LPG future equipment if still shown, show current NGL/stabilizer/SOC/IA/heat-medium package locations, and clarify the shared incinerator relationship to the 3-25 location.

Evidence used: PDF text extraction found stabilizer, SOC, heat medium, incinerator, IA, LPG, depropanizer, and mole sieve terms; D4-SCA001 through D4-SCA006 modify those bases.

Review caveats: CAD source review is required before issuing a revised sheet; coordinate sheet 5002 with sheet 5001.

## 15. MFS-235633 (4-25 Doe)_P&ID_rA.pdf

Why this document is impacted: P&IDs carry all detailed process/utility/relief/control changes.

Specific expected revisions: Revise all affected sheets for NGL, no depropanizer, non-regenerative caustic, stabilizer/SOC, inlet HEX, heat medium, TEG/mole sieve, IA, incinerator and electrical changes.

Evidence used: The regulatory PDF is the 4-25 P&ID package; D4-SCA001 through D4-SCA006 alter systems shown on P&IDs.

Review caveats: Requires discipline drawing-by-drawing review.

## 16. MSS-235633-1001 (4-25 Doe)_Metering Schematic_rB.pdf

Why this document is impacted: Product nomenclature and product transfer bases changed.

Specific expected revisions: Confirm all LPG/C3-C4/depropanizer metering labels are replaced or retired and NGL basis is current.

Evidence used: The regulatory PDF is the 4-25 metering schematic; D4-SCA001 and D4-SCA003 alter product terminology and transfer/metering basis.

Review caveats: May be no-change if it only covers unaffected utility/gas metering.

## 17. PFD-235633 (4-25 Doe)_Process Flow Diagram_rB.pdf

Why this document is impacted: Flow paths and equipment bases changed throughout the process.

Specific expected revisions: Update all process flows for NGL, removed depropanizer, non-regenerative caustic, stabilizer/SOC, inlet HEX, heat medium, TEG/mole sieve, IA, and incinerator basis.

Evidence used: The regulatory PDF is the 4-25 process flow diagram package; D4-SCA001 through D4-SCA006 alter represented process flows and utility interfaces.

Review caveats: Coordinate with revised winter/summer HMBs.

## 18. Surface Water Management Plan_rB.pdf

Why this document is impacted: Layout and containment assumptions may change.

Specific expected revisions: Review drainage/containment around NGL, heat-medium, IA, stabilizer/SOC and removed depropanizer/LPG areas.

Evidence used: PDF text hits LPG/NGL/heat medium/mole sieve; D4-SCA001 and D4-SCA002 alter layout and drainage-relevant equipment bases.

Review caveats: Requires revised plot plan.

## 19. Light Assessment_rC.pdf

Why this document is impacted: Lighting inventory and narrative may include changed/removed equipment.

Specific expected revisions: Update lighting sources and project narrative for current 4-25 package layout.

Evidence used: PDF text hits LPG/NGL/heat medium/mole sieve; D4-SCA001 and D4-SCA006 alter package layout and lighting-source assumptions.

Review caveats: If fixtures unchanged, narrative-only update may be sufficient.

## 20. Ground Permissive at Truck-Out_rB.pdf

Why this document is impacted: Product loading/truck-out terminology may be stale after NGL/LPG changes.

Specific expected revisions: Review product labels, truck-out interlocks, and grounding philosophy against current NGL loading scope.

Evidence used: PDF text hits LPG/NGL/truck-out; D4-SCA001 and D4-SCA003 alter product terminology and transfer/loading basis.

Review caveats: May remain unchanged if grounding hardware is product-agnostic.

## 21. W235633-PRC-DBM-000001-001 (4-25 Doe)_Design Basis Memorandum_rA.pdf

Why this document is impacted: It contains all major superseded 4-25 basis items.

Specific expected revisions: Rewrite DBM sections for NGL scope, depropanizer removal, non-regenerative treating, stabilization/SOC, heat medium, inlet HEX, TEG/mole sieve, IA, incinerator, electrical, and package roster.

Evidence used: PDF text hits every major changed term; SCA report is DBM amendment basis.

Review caveats: Should precede dependent regulatory report revisions.

## 22. W235633-PRC-HMB-000001-001 (4-25 Doe)_HMB Winter_rC.pdf

Why this document is impacted: The winter material balance changes with depropanizer removal, NGL treating, heat medium, IA, and stabilizer/SOC decisions.

Specific expected revisions: Recalculate the winter case for the current process configuration, NGL product basis, DSO/spent-caustic waste basis, stabilizer/SOC loads, inlet HEX relationship, heat-medium loads, and incinerator/flare feed basis.

Evidence used: The winter HMB regulatory PDF text hits LPG, SOC, incinerator, VRU, TEG, and spent caustic; D4-SCA001 through D4-SCA006 alter those process bases.

Review caveats: Winter operating assumptions should be checked separately from summer assumptions before dependent emissions and flare calculations are revised.

## 23. W235633-PRC-HMB-000002-001 (4-25 Doe)_HMB Summer_rC.pdf

Why this document is impacted: The summer material balance changes with depropanizer removal, NGL treating, heat medium, IA, and stabilizer/SOC decisions.

Specific expected revisions: Recalculate the summer case for the current process configuration, NGL product basis, DSO/spent-caustic waste basis, stabilizer/SOC loads, inlet HEX relationship, heat-medium loads, and incinerator/flare feed basis.

Evidence used: The summer HMB regulatory PDF text hits LPG, SOC, incinerator, VRU, TEG, and spent caustic; D4-SCA001 through D4-SCA006 alter those process bases.

Review caveats: Summer operating assumptions should be checked separately from winter assumptions before dependent emissions and flare calculations are revised.

## 24. Emissions Summary_rB.pdf

Why this document is impacted: Emission sources and rates changed materially.

Specific expected revisions: Recalculate source inventory for current NGL, no depropanizer, non-regenerative caustic, heat medium, IA, incinerator, flare and stabilizer/SOC basis.

Evidence used: PDF text hits changed emissions sources; D4-SCA001 through D4-SCA006 alter equipment, utility, incinerator, flare, and emissions bases.

Review caveats: Must follow revised HMB, flare data, and equipment list.

## 25. Non-Flare Connected PSV Summary_rA.pdf

Why this document is impacted: Relief basis may include retired or renamed systems.

Specific expected revisions: Remove retired depropanizer/LPG entries, update NGL/stabilizer/SOC/heat-medium/IA PSV basis, and confirm flare/non-flare destinations.

Evidence used: PDF text hits depropanizer/LPG/NGL/IA/heat medium; D4-SCA001 through D4-SCA006 alter PSV and relief-source bases.

Review caveats: Requires revised P&IDs and relief register.

## 26. Inlet Separator Summary.pdf

Why this document is impacted: Inlet separator basis and HEX subject changed.

Specific expected revisions: Update installed count/sizing, future third plot, liquid outlet heater, and downstream MPFF/stabilizer interface.

Evidence used: The regulatory PDF is the 4-25 inlet separator summary; D4-SCA001 changes the separator basis and D4-SCA005 registers the inlet-separator HEX subject.

Review caveats: Confirm final heat source for HEX.

## 27. Prime Mover List_rC.pdf

Why this document is impacted: Driver/motor lists change with VFD, IA, heat medium and process-scope changes.

Specific expected revisions: Update motors/drivers for current stabilizer/SOC, heat medium, TEG/mole sieve, instrument air, VFD, capacitor-bank removal and standby power basis; remove depropanizer/LPG items.

Evidence used: PDF text hits LPG/NGL/stabilizer/heat medium/VFD/mole sieve; D4-SCA001 and D4-SCA006 alter prime-mover, drive, and electrical assumptions.

Review caveats: Final electrical studies may change ratings.

## 28. Plant Shutdown & Blowdown Philosophy_rB.pdf

Why this document is impacted: Shutdown/blowdown basis depends on current equipment, relief and utility configuration.

Specific expected revisions: Update shutdown/blowdown paths and logic for NGL, no depropanizer, heat medium, IA, incinerator, and stabilizer/SOC scope.

Evidence used: PDF text hits depropanizer/LPG/NGL/heat medium/incinerator/IA; D4-SCA001 through D4-SCA006 alter shutdown, blowdown, relief, utility, and interface assumptions.

Review caveats: Coordinate with revised P&IDs and flare data.

## 29. Flare Stack Data Submission_rA.pdf

Why this document is impacted: Relief and incinerator loads changed.

Specific expected revisions: Remove caustic-regeneration/depropanizer relief bases, update incinerator feed and location, and recalculate flare load cases.

Evidence used: SCA-002 Supersession_Delta and PDF text hits caustic regeneration/incinerator/LPG/depropanizer.

Review caveats: Requires process/relief engineer signoff.

## 30. DEOS_rB.pdf

Why this document is impacted: It may contain dehydration or incinerator assumptions affected by SCAs.

Specific expected revisions: Confirm retained dehydration scope and remove obsolete LPG/depropanizer or caustic-regeneration/incinerator statements if present.

Evidence used: PDF text hits dehy/SOC/incinerator/flare/VRU/TEG.

Review caveats: May be no-change if strictly limited to retained equipment.

## 31. Safety & Loss Management_rA.pdf

Why this document is impacted: Safety responsibilities and safeguards may change with shared and retired systems.

Specific expected revisions: Review safety narrative, safeguards, shared incinerator operation, IA failure modes, heat medium changes, and current NGL hazards.

Evidence used: PDF text hits LPG/NGL/stabilization/SOC/heat medium/flare/mole sieve; D4-SCA001 through D4-SCA006 alter safety-relevant process and utility bases.

Review caveats: Should follow updated HAZID.

## 32. HAZMAT List_rC.pdf

Why this document is impacted: Hazardous materials inventory is directly affected by NGL, non-regenerative caustic, heat medium, TEG/mole sieve and IA changes.

Specific expected revisions: Replace LPG/depropanizer-specific entries, update NGL/DSO/spent-caustic, heat-medium, TEG, mole sieve and IA consumables.

Evidence used: PDF text hits LPG/NGL/heat medium/IA/TEG/mole sieve/spent caustic; D4-SCA001 through D4-SCA006 alter the material inventory basis.

Review caveats: Final quantities require updated HMB and tank lists.

## 33. QAP & Validation Report_rA.pdf

Why this document is impacted: Package/decomposition structure changed through SCA-004/SCA-005.

Specific expected revisions: Check validation references, package roster, and remediation status.

Evidence used: The regulatory PDF is the 4-25 QAP and validation report; D4-SCA004 and D4-SCA005 change package roster and remediation validation context.

Review caveats: Likely administrative unless report embeds technical package lists.

## 34. Secondary Containment Plan_rB.pdf

Why this document is impacted: Containment assumptions change with product, tank and chemical scope.

Specific expected revisions: Update containment around current NGL/DSO/spent caustic, heat medium, tanks, removed depropanizer/LPG, and shared incinerator interface.

Evidence used: PDF text hits depropanizer/LPG/NGL/heat medium/incinerator/spent caustic; D4-SCA001 through D4-SCA006 alter containment-relevant equipment and material bases.

Review caveats: Requires revised plot and HAZMAT list.

## 35. Adopted Codes & Standards_rB.pdf

Why this document is impacted: Equipment-specific standards may need current package alignment.

Specific expected revisions: Review standards for retired depropanizer/LPG systems and current NGL, heat medium, mole sieve, IA, incinerator and electrical packages.

Evidence used: PDF text includes stale scope terms; D4-SCA001 through D4-SCA006 and the package roster changes alter the standards-review basis.

Review caveats: May be no-change if standards are generic.

## 36. Facility Security & Fire Protection_rC.pdf

Why this document is impacted: Layout, hazardous inventory and shared asset responsibility changed.

Specific expected revisions: Review fire/security coverage for NGL/tanks, heat medium, IA, incinerator, removed depropanizer, and changed package layout.

Evidence used: PDF text hits LPG/NGL/heat medium/flare/mole sieve; D4-SCA001 through D4-SCA006 alter hazardous inventory, fire-protection, and layout assumptions.

Review caveats: Requires updated HAZMAT, plot and consequence basis.

## 37. HAZID Study Report_r0.pdf

Why this document is impacted: HAZID scope and scenarios changed materially.

Specific expected revisions: Reconcile nodes, scenarios, recommendations, document references and risk rankings with all SCA decisions.

Evidence used: PDF text hits all major changed terms.

Review caveats: Requires formal HAZID addendum or revalidation workshop.

## 38. Prelim Consequence Assessment_rA.pdf

Why this document is impacted: Consequence scenarios depend on current equipment, inventory, relief and incinerator basis.

Specific expected revisions: Re-run scenario basis for NGL, no depropanizer, current heat medium/IA, shared incinerator and changed relief/blowdown.

Evidence used: PDF text hits LPG/NGL/heat medium/mole sieve/stabilization; D4-SCA001 through D4-SCA006 alter consequence-relevant process, utility, and relief bases.

Review caveats: Should follow revised HAZID and flare data.

## 39. Design Change Summary_rB.pdf

Why this document is impacted: The submitted design change summary predates the full SCA chain.

Specific expected revisions: Replace with or append a consolidated SCA-001 through SCA-006 change summary covering product, process, instrument air serving both facilities, incinerator, standby/VFD/capacitor electrical changes, and package-structure changes.

Evidence used: PDF text hits depropanizer/LPG/NGL/heat medium/incinerator/LACT/mole sieve.

Review caveats: This may become the control document for the regulatory amendment package; use clear revision traceability.

# Evidence Limits and Open Review Items

PDF text extraction was sufficient for most narrative reports, but drawing PDFs should be checked visually against CAD/source drawings before final revision instructions are issued.

Several package deliverables identify missing source slices, especially vendor RFQ/DOCX references. This report uses the package context as derivative planning evidence, not as authoritative vendor-source truth.

SCA evidence establishes scope decisions but does not supply all recalculated values. HMB, emissions, flare, noise, PSV, consequence and shutdown/blowdown documents require discipline recalculation.

Incinerator operational responsibility remains an open coordination item on the 4-25 side and should be resolved before final flare/incinerator regulatory submissions.

TEG contactor sparing and NGL regeneration gas/source details remain open or conflict-marked in 4-25 SCA-005 evidence and related package deliverables; regulatory reports should preserve those as review items until ruled.

The no-change classifications apply only to the SCA-driven scope review performed here. They do not certify currency, signature status, regulatory sufficiency, or document-control validity.